Mission Manager Subsystem

Technical Reference: New Files, What Each One Does, and What Needs to Change Next

Drone Formation Interface — formation_interface / formation_interfaces ROS 2 packages
Generated 2026-08-10

# Table of Contents

Right-click and choose "Update Field" to build the table of contents.

# 1. Overview & Context

This document describes a new “mission manager” subsystem added to the drone formation interface project, built to satisfy a set of flight-operations requirements from the project's requirements list: define a flight profile independently of any one drone, define a drone's own parameters independently of any one mission, assign drones to flights through a GUI, start and interrupt (gently) one or all drones, monitor live swarm health, and deploy configuration to — and retrieve logs from — each drone.

The mission manager is a second, independent layer added on top of the existing formation-flying stack (formation_node.py / drone_commander.py / formations.py). It does not replace that stack and does not change its behavior — the only change made to existing code is one additive keyword argument on drone_commander.py, fully backward compatible. This mirrors how the existing live-monitor subsystem (active_tracker.py / monitor_gui.py) already sits alongside formation_node.py as an independent layer with its own topics, its own GUI, and no shared state.

## 1.1 Architecture at a Glance

HOST PC                                          DRONE (per drone_profiles/<name>.yaml)
+-----------------------------+                  +-----------------------------------+
| mission_gui.py               |  ExecuteFlight   | mission_node.py                    |
|   MissionApp (Tk)             |  (action, one   |   ActionServer, one goal PER DRONE |
|   create/select mission ------+->  goal per   ->|   resolves flight profile          |
|   assign flight per drone     |    drone_name)   |   picks a deploy backend:          |
|   Start / Interrupt / Terminate                  |     local    -> drone_commander    |
|   <- live DroneTelemetry table ------------------|     ssh      -> real companion PC  |
+-----------------------------+  TerminateFlight   |     mock_ssh -> simulated, no HW   |
                                  (service)          |   PX4TelemetryWatcher per drone    |
                                                     |   publishes /mission/drone_telemetry
                                                     +-----------------------------------+
                                                                    |
                                                       backend: ssh |  scp params, ssh start,
                                                                    v  ssh SIGINT to interrupt,
                                                     +-----------------------------------+
                                                     | flight_executor_node.py             |
                                                     |   (companion computer, launched      |
                                                     |    remotely by SSHDeployer)          |
                                                     |   reads rendered params.yaml          |
                                                     |   drives PX4Commander for real         |
                                                     +-----------------------------------+

The deploy backend is a pluggable strategy — chosen per drone, per mission run — mirroring the existing backend:=sim / backend:=px4 pattern already used by drone_commander.py. Three backends satisfy the same interface: local (no SSH, the mission node streams setpoints in-process), ssh (real per-drone companion computer deploy over the system ssh/scp/rsync binaries), and mock_ssh (the default — zero network required, fully exercises the Start / telemetry / Interrupt / Terminate flow with simulated state).

## 1.2 Requirement → Implementation Map

| Requirement | Implemented in |
| --- | --- |
| Flight profile (state / control scheme / shared params / trajectory), drone-agnostic | flight_profiles/*.yaml, flight_profiles.py |
| Drone parameters, drone-specific, not mission-specific | config/drone_profiles/*.yaml, drone_profiles.py |
| Assign drones to a flight; GUI; quickly varying | mission_gui.py (assignment table) |
| Start button, one or all drones | mission_gui.py buttons → ExecuteFlight action goals |
| Interrupt button, one or all drones, gentle | ExecuteFlight goal cancel → deployer.interrupt(hard=False) |
| Live monitoring: battery, error flags, mode, tracking error | DroneTelemetry.msg, telemetry.py, mission_node.py publisher, mission_gui.py table |
| Sensor handshakes, remote connection, clear drone mapping, capability | DroneTelemetry fields pre_flight_checks_pass / gcs_connection_lost / link_ok; string drone_name identity; drone_profiles.py capabilities |
| Mission name / description, README, no accidental overwrites | mission_store.py |
| Activate / deactivate features | mission_gui.py checkboxes → mission.yaml features |
| Trigger drones: generate config, copy to drone, execute | deploy/launch_gen.py + deploy/backends.py: SSHDeployer |
| Terminate: copy local logs back to correct path | TerminateFlight.srv, deploy/backends.py terminate() methods |

# 2. New Interfaces — formation_interfaces Package

formation_interfaces is the ament_cmake package holding all custom .action / .msg / .srv definitions (ROS 2 requires code-generated interfaces to live in a CMake package, separate from the ament_python nodes that use them). Three new files were added here, alongside the pre-existing GoToFormation.action, DronePose.msg, and DroneActive.msg.

## 2.1 action/ExecuteFlight.action

Commands ONE drone (identified by its drone_profiles/<name>.yaml name, not an array index) to execute a flight profile as part of a mission. Unlike GoToFormation — which is one goal for the whole swarm — mission_node accepts one concurrent goal PER drone name, so drones start and interrupt independently of each other.

| Section | Field | Type | Meaning |
| --- | --- | --- | --- |
| Goal | mission_name | string | Must name an existing missions/<mission_name>/ directory. |
| Goal | drone_name | string | Matches a config/drone_profiles/<name>.yaml `name:` field. |
| Goal | flight_profile | string | Optional override; empty resolves the profile from the mission's assignment map instead. |
| Goal | backend | string | Empty (use drone profile default) ¦ local ¦ ssh ¦ mock_ssh. |
| Result | success | bool |  |
| Result | message | string |  |
| Result | phase | string | Final phase reached: held ¦ landed ¦ error. |
| Feedback | phase | string | deploying ¦ copying ¦ starting ¦ streaming ¦ holding ¦ done ¦ error. |
| Feedback | nav_state | uint8 | Mirrors px4_msgs VehicleStatus.nav_state; 255 if unknown. |
| Feedback | arming_state | uint8 | Mirrors px4_msgs VehicleStatus.arming_state; 0 if unknown. |
| Feedback | pre_flight_checks_pass | bool | Sensor handshake. |
| Feedback | gcs_connection_lost | bool | Remote connection status. |
| Feedback | battery_remaining | float32 | 0..1, -1 if unknown. |
| Feedback | tracking_error | float32 | Metres, -1 if unknown. |

Design note:  Cancelling a goal is the gentle Interrupt for that one drone: the cancel callback accepts, the execute loop notices is_cancel_requested and calls deployer.interrupt(hard=False) — freezing the last commanded setpoint, the same free “hold in place” behavior formation_node already gets today by simply not refreshing a target after cancel — then calls goal_handle.canceled(). It does NOT land or retrieve logs; that is the separate TerminateFlight service, described next.

## 2.2 srv/TerminateFlight.srv

| Section | Field | Type | Meaning |
| --- | --- | --- | --- |
| Request | mission_name | string |  |
| Request | drone_name | string | Empty = every drone currently active under this mission. |
| Request | land_first | bool | True = attempt AUTO_LAND before pulling logs. |
| Response | success | bool |  |
| Response | message | string |  |
| Response | log_paths | string[] | Local paths logs were copied to, one per drone processed. |

Interrupt and Terminate were deliberately split into two separate operations because the requirements list them separately: Interrupt is the fast, safe “pause” button usable mid-mission without ending it; Terminate is the deliberate end-of-flight action that lands (best-effort) and always retrieves logs.

## 2.3 msg/DroneTelemetry.msg

One drone's live mission status, published on a single shared topic (/mission/drone_telemetry) tagged by drone_name — a string identity, not the int32 drone_id used by the pre-existing DronePose/DroneActive messages. This is a deliberate divergence: the “clearly map to a drone” and “quickly varying assignment” requirements argue against a fixed-length-array index as the identity.

| Field | Type | Meaning |
| --- | --- | --- |
| drone_name | string | Identity — matches a drone_profiles/<name>.yaml `name:`. |
| nav_state | uint8 | Local copy of px4_msgs VehicleStatus.nav_state enum values (NAV_STATE_UNKNOWN=255, OFFBOARD=14, AUTO_LOITER=4, AUTO_LAND=18, AUTO_RTL=5). |
| arming_state | uint8 | Local copy of ARMING_STATE (UNKNOWN=0, DISARMED=1, ARMED=2). |
| pre_flight_checks_pass | bool | Sensor handshake. |
| gcs_connection_lost | bool | Remote connection status. |
| failsafe_active | bool |  |
| link_ok | bool | Deploy-backend-level liveness (see §13 for its current limits). |
| battery_remaining | float32 | 0..1, -1 if unknown. |
| battery_voltage_v | float32 | -1 if unknown. |
| tracking_error | float32 | Metres, -1 if unknown. |
| mission_name | string | Empty if no mission is active for this drone. |
| mission_phase | string | idle ¦ deploying ¦ copying ¦ starting ¦ streaming ¦ holding ¦ terminating ¦ done ¦ error. |
| error_message | string | Empty if none (not populated by anything yet — see §13). |

The PX4 enum values are kept as local constants in this message (and in flight_profiles.py's NAV_STATE_NAMES) rather than importing px4_msgs directly, so this message and any pure-Python code that reads it stays usable with zero px4_msgs installed — matching the mock-backend design goal.

## 2.4 CMakeLists.txt (edited)

The three new files were registered alongside the two existing messages and the existing action:

rosidl_generate_interfaces(${PROJECT_NAME}
  "action/GoToFormation.action"
  "action/ExecuteFlight.action"
  "msg/DronePose.msg"
  "msg/DroneActive.msg"
  "msg/DroneTelemetry.msg"
  "srv/TerminateFlight.srv"
  DEPENDENCIES geometry_msgs
)

package.xml for formation_interfaces was left unchanged — none of the new interfaces introduce a new external message dependency.

# 3. ROS-Free Core Modules

Three modules were added with zero ROS imports, following the same pattern already established by formations.py and planning.py: pure Python, dataclass-based, directly unit-testable with plain pytest and no workspace sourcing required. They are imported by both mission_node.py (ROS-dependent) and mission_gui.py (ROS-dependent), and by their own test files.

## 3.1 flight_profiles.py

Defines a flight profile: a target PX4 flight state, a control scheme, a bag of non-drone-specific parameters, and a time-parametric reference trajectory. Says nothing about which drone flies it, or that drone's namespace / system_id / capabilities — that is drone_profiles.py's job.

| Name | Kind | Purpose |
| --- | --- | --- |
| NAV_STATE_NAMES | dict | Name → int, mirrors px4_msgs VehicleStatus.msg's NAVIGATION_STATE_* constants. Hand-kept in sync — see §13. |
| CONTROL_SCHEMES | tuple | ("position", "velocity", "trajectory") |
| FlightProfile | dataclass | name, description, state, control_scheme, params (dict), trajectory (dict), required_capabilities (list). |
| load_flight_profile(path) | function | Parses + validates one YAML file. Raises ValueError on an unknown state or control_scheme. |
| list_flight_profiles(dir_) | function | Loads every *.yaml in a directory, keyed by each profile's `name` field (not filename). Raises ValueError on a duplicate name. |
| sample_trajectory(spec, t, base) | function | Returns (x, y, z, yaw) in world ENU at time t seconds since flight start, relative to base. Dispatches on spec['type']. |

Three trajectory types are supported:

static_point  — hold a fixed offset from base indefinitely. Fields: offset [dx, dy, dz], optional yaw.

circle  — fly a level circle. Fields: radius, period (seconds for one full revolution), offset. Yaw faces tangent to the circle.

waypoints  — linear interpolation between explicit [x, y, z, t] points, clamped before the first and after the last point. See §13 for a known ROS-parameter limitation with this type.

Example flight profile YAML (hover_low.yaml, shipped as a starting example):

name: hover_low
description: >
  Point-hold smoke test: climb to a fixed altitude above home and hold. No
  formation offset - this is a single-drone flight profile, not tied to any
  particular swarm shape.
version: 1
state: OFFBOARD
control_scheme: position
params:
  altitude: 1.5
  position_tolerance: 0.15
  settle_cycles: 10
  goal_timeout: 60.0
  yaw: 0.0
trajectory:
  type: static_point
  offset: [0.0, 0.0, 1.5]
required_capabilities: []

## 3.2 drone_profiles.py

Defines a drone's own parameters that are drone-specific but NOT mission-specific: namespace, system id, pose topic, capability tags, speed/accel limits, default deploy backend, and (for the ssh backend) companion-computer connection details.

Design decision:  one file per physical drone under config/drone_profiles/, rather than extending config/drones.yaml's index-aligned parallel-list shape. Rationale: “quickly varying assignment” means adding, editing, or retiring one drone should touch exactly one small file, not risk an off-by-one across several parallel arrays; capability matching needs a natural per-drone list; and the drone count becomes dynamic (the loader globs the directory) instead of fixed by array length. config/drones.yaml itself is left completely untouched — it keeps serving formation_node.py / gui_node.py exactly as before. The two are NOT kept in sync automatically (see §13).

| Name | Kind | Purpose |
| --- | --- | --- |
| DroneProfile | dataclass | name, namespace, system_id, pose_topic, pose_topic_type, capabilities (list), max_speed, max_accel, backend, ssh_host, ssh_user, ssh_key_path, remote_workspace_path, remote_log_dir. |
| load_drone_profile(path) | function | Parses + validates one YAML file. Raises ValueError if a required field (name/namespace/system_id/pose_topic) is missing. |
| list_drone_profiles(dir_) | function | Loads every *.yaml in a directory, keyed by `name`. Raises ValueError on a duplicate name. |
| drone_supports(profile, required) | function | True if `required` (a list of capability strings) is a subset of the profile's capabilities. |

Example drone profile YAML (drone1.yaml, matching config/drones.yaml's real entry):

name: drone1
namespace: px4_1                          # matches config/drones.yaml's drone_namespaces[0]
system_id: 2                              # matches config/drones.yaml's system_ids[0]
pose_topic: "/vrpn_mocap/drone1/pose"     # matches config/drones.yaml's pose_topics[0]
pose_topic_type: PoseStamped
capabilities: ["gps"]
max_speed: 2.0                            # m/s
max_accel: 1.5                            # m/s^2
backend: mock_ssh                         # local | ssh | mock_ssh - safe default, no hardware
ssh_host: ""                              # >>> set for backend: ssh <<<
ssh_user: ""
ssh_key_path: ""
remote_workspace_path: "/home/px4/formation_ws"
remote_log_dir: "/home/px4/formation_ws/logs"

## 3.3 mission_store.py

A mission pairs a name/description with an assignment of drone_name → flight_profile name, plus feature toggles. Written once under missions/<name>/ and never silently overwritten.

| Name | Kind | Purpose |
| --- | --- | --- |
| MissionExistsError | exception | Raised by create_mission() when the mission directory already exists and force was not given — the entire anti-overwrite mechanic is this one plain directory-existence check. |
| MissionSpec | dataclass | name, description, created_at, author, assignment (dict), features (dict). |
| DEFAULT_FEATURES | dict | {"auto_arm": False, "pull_logs_on_terminate": True, "record_bag": False} |
| create_mission(root, name, ...) | function | mkdir missions/<name>/; writes mission.yaml + README.md. Raises MissionExistsError unless force=True. |
| load_mission / save_mission | functions | save_mission() overwrites mission.yaml only — it never touches README.md. |
| list_missions(root) | function | Every subdirectory that has a mission.yaml. |
| write_readme_if_absent(dir, spec) | function | Writes README.md only if it does not already exist. |
| new_run_dir(mission_dir) | function | A fresh runs/<UTC-timestamp>/ directory, never reused — retried with a -2, -3… suffix on a same-second collision. |

Example mission.yaml, as written by create_mission()/save_mission():

name: tuesday_wind_test
description: >
  Baseline hover-and-hold test, drones 1 and 2 only.
created_at: "2026-08-09T14:03:00Z"
author: ""
assignment:
  drone1: hover_low
  drone2: circle_survey
features:
  auto_arm: false
  pull_logs_on_terminate: true
  record_bag: false

Runtime data layout under missions/ (repo root, sibling to bag_data/):

missions/<mission_name>/
  mission.yaml        # assignment + features, overwritable (never silently)
  README.md            # written once, never regenerated
  runs/<UTC-timestamp>/
    params/<drone_name>_params.yaml   # what was rendered/copied to the drone
    logs/<drone_name>/...              # pulled back on Terminate

mission.yaml and README.md are small and git-trackable; runs/ (per-run params and logs) is added to .gitignore.

# 4. Deploy Backend System

The deploy package (formation_interface/deploy/) answers the requirement “trigger drones to execute a particular flight — generate a launch file from drone name and flight profile, copy to drone, execute controller mode” and “terminate: tell the drone to copy any local logs to the main computer.” It is a pluggable-backend design mirroring drone_commander.make_commander()'s existing factory + lazy-import pattern exactly, so mission_node.py never branches on which backend is active — every backend exposes the same precheck() / deploy() / interrupt() / terminate() / phase contract.

## 4.1 deploy/launch_gen.py

Pure functions, no ROS dependency, no side effects beyond the one explicit file write.

| Function | Purpose |
| --- | --- |
| render_single_drone_params(drone_profile, flight_profile, mission_name, run_id) | Pure — builds the flat parameter dict a companion computer's flight_executor_node.py needs (namespace, system_id, state, control_scheme, trajectory, tolerances, etc.). |
| write_params_yaml(params, dest_path) | Writes that dict wrapped in the drones.yaml-style /**: ros__parameters: wildcard shape. This file, unlike the profile/mission YAMLs above, IS actually loaded via declare_parameter, by flight_executor_node.py. |

Design note:  rather than templating and regenerating an actual .launch.py file per mission (fragile — it would mean executing freshly generated code), the concrete artifact “generated and copied” is a rendered params YAML. single_drone_flight.launch.py (§6) is a static, checked-in launch file that just accepts that file as a params_file argument.

## 4.2 deploy/backends.py

def make_deployer(backend, node, drone_profile, mission_name, run_dir):
    if backend == "local":
        return LocalDeployer(...)
    if backend == "ssh":
        return SSHDeployer(...)
    return MockSSHDeployer(...)   # default for '', 'mock_ssh', or anything unrecognised

LocalDeployer  (backend: local)

No SSH, no companion computer. deploy() calls drone_commander.make_commander("px4", ...) so mission_node's own control loop streams setpoints in-process — exactly what formation_node.py already does. interrupt() just marks phase="holding" (the setpoint is frozen by the caller, not re-issued). terminate() has no companion computer to pull logs from, so it returns success with log_path=None. land() is available via the underlying PX4Commander (see §5.5) for the land_first Terminate option.

SSHDeployer  (backend: ssh)

Real per-drone companion-computer deploy, using the system's own ssh / scp / rsync binaries via Python's subprocess module — no new pip dependency was added. Companion computers today run individual loose Python scripts (per from_drone/mocap_update.py), not a full colcon workspace install, so what gets copied is one small rendered params file, not a package install.

| Method | What it actually shells out to |
| --- | --- |
| precheck() | ssh -o BatchMode=yes -o ConnectTimeout=5 <host> true |
| deploy() | 1) render + write the params YAML locally 2) scp <params>.yaml <user>@<host>:<remote_workspace_path>/ 3) ssh <host> "nohup ros2 launch formation_interface single_drone_flight.launch.py params_file:=<path> > <log> 2>&1 & echo $!" (captures the remote PID) |
| interrupt(hard) | ssh <host> "kill -SIGINT <pid>" (or -SIGKILL if hard=True) |
| terminate() | interrupt(hard=False) if still running, then rsync -az <host>:<remote_log_dir>/ <local run_dir>/logs/<drone_name>/ |

Status:  written and structurally complete, but NOT yet validated against real hardware — no companion computer was available during this work. MockSSHDeployer satisfies the identical interface for dry runs; switching a drone profile's backend: from mock_ssh to ssh is the only change needed once a companion computer is reachable.

MockSSHDeployer  (backend: mock_ssh — the default)

Zero real network. Same shape as the pre-existing mock_optitrack.py: internal simulated state advanced by a repeating rclpy timer instead of real subprocess calls, so the whole Start → deploy → telemetry → Interrupt → Terminate → log-pull flow is demoable and testable with no hardware at all. One instance exists per active goal (embedded as a plain class driven by a timer on the passed-in mission_node), not a separate process.

| Phase sequence (one step per 0.5 s timer tick) |  |
| --- | --- |
| deploying → copying → starting → streaming → holding | (reached via interrupt(), or by staying at streaming) |

telemetry_sample() returns a fake VehicleStatus-shaped sample (OFFBOARD / ARMED once streaming, 85% battery, 15.8 V) so the GUI's telemetry table has something realistic to show without px4_msgs. terminate() writes a real mock.ulog file into the run's logs/<drone_name>/ directory and cancels its own timer.

# 5. ROS-Dependent Nodes & GUI

## 5.1 telemetry.py

TelemetrySample is a plain dataclass with no ROS dependency, so MockSSHDeployer can construct one without importing px4_msgs. PX4TelemetryWatcher is the real backend: it subscribes to one drone's VehicleStatus and BatteryStatus on /<namespace>/fmu/out/..., lazily importing px4_msgs inside __init__ — exactly mirroring PX4Commander's own lazy-import pattern in drone_commander.py, so mock-only setups and unit tests never need px4_msgs installed. One watcher is created per configured drone at mission_node startup, independent of any goal — idle drones still report live battery/nav_state.

## 5.2 mission_node.py

The action server and service host. Structurally similar to formation_node.py (a ReentrantCallbackGroup, an ActionServer, a control timer that must never stall) but with one key difference: it accepts ONE CONCURRENT GOAL PER drone_name instead of one goal for the whole swarm. formation_node.py's goal callback rejects any new goal while self._active is True (i.e. only one drone-group operation at a time, system-wide); mission_node.py's goal callback only rejects a new goal for a drone that already has one — every other drone remains free to start/stop independently.

Startup sequence:

Declares parameters: missions_root, drone_profiles_dir, flight_profiles_dir, default_backend, telemetry_rate, telemetry_stale_timeout, goal_timeout.

Loads every drone profile and every flight profile via list_drone_profiles() / list_flight_profiles().

Subscribes to each drone's pose topic (for tracking-error calculation) and creates one PX4TelemetryWatcher per drone, independent of goal state.

Creates the /mission/drone_telemetry publisher and a timer that publishes one message per configured drone at telemetry_rate Hz.

Creates the ExecuteFlight ActionServer and the TerminateFlight service.

Per-goal execution (_run_goal), phase by phase:

Resolve the flight profile — either the goal's explicit override, or by looking up the mission's assignment map for that drone_name.

Check drone_supports() — abort if the drone lacks a capability the flight profile requires.

Pick the backend (goal override → drone profile default → node default) and construct a deployer via make_deployer().

precheck(), then deploy() — abort on either returning False.

Store the deployer in self._deployers[drone_name] — from this point it outlives the goal callback itself; it is what keeps the drone streaming or holding after the goal concludes, until an explicit Terminate.

Loop at 5 Hz: check for a cancel request (→ gentle interrupt, described above), publish feedback (phase, PX4 mode/arming, sensor flags, battery, live tracking error), and watch the deployer's phase.

Bug found and fixed during end-to-end testing:  the goal originally called goal_handle.succeed() as soon as the deployer reached the “streaming” phase. That meant a drone that had settled into “holding” no longer had an open goal for the operator to cancel — Interrupt would silently do nothing on exactly the drones an operator would actually want to interrupt (mid-flight, not mid-deploy). This was caught by a live rclpy test that confirmed the goal must still be open one second after reaching “streaming”. Fixed by removing the early success() call: the goal now stays open for the ENTIRE streaming/holding duration and only concludes when the client cancels it (gentle interrupt, result phase="held"), when an external TerminateFlight call lands and clears the deployer while the goal is still open (result phase="landed"), or when the deploy transition itself times out or errors before ever reaching streaming.

TerminateFlight service handler:

Selects every drone matching the request's mission_name (if given) and drone_name (if given; empty means every active drone under that mission).

For each match, optionally attempts a land command (see §5.5), then calls deployer.terminate() — which lands/stops the remote process as needed and pulls logs back — and removes the drone from self._deployers / self._flight_ctx.

Returns one aggregate response: overall success, a semicolon-joined per-drone message, and the list of local log paths retrieved.

## 5.3 mission_gui.py

Tkinter control panel. Uses the exact same two-thread model already established by gui_node.py and monitor_gui.py: rclpy spins on a daemon thread, Tk owns the main thread, and ROS callbacks never touch Tk directly — they push events onto a queue.Queue drained by a 10 Hz after() poll. The live telemetry table is read directly from a plain dict (atomic under the GIL), the same pattern gui_node.py's live pose table already uses.

| Panel | Contents |
| --- | --- |
| Mission | Name/description entry, “Create Mission” button (calls create_mission() inline — fast local disk I/O, no queue needed, same “safe to call directly from a Tk handler” precedent as monitor_gui's plan_for()); on MissionExistsError, an overwrite-confirm dialog. A combobox to pick an existing mission, plus a “reload profiles” button. |
| Assignment | One row per drone profile: name, capability tags, and a flight-profile dropdown filtered to profiles that drone actually supports (drone_supports()). Per-row Start / Interrupt / Terminate buttons. |
| All drones | Start ALL, INTERRUPT ALL (gentle), a “land first” checkbox, and Terminate & Retrieve Logs (ALL). |
| Features | Checkboxes bound to mission.yaml's features dict, saved immediately on toggle. |
| Live telemetry | A ttk.Treeview table: one row per drone — phase, PX4 mode label, armed, battery %, tracking error, sensor-checks pass/fail, link OK/LOST, current mission. |

Per-row Start/Interrupt buttons are gated on whether that drone currently has an entry in the backend's goal-handle dict — Start disabled and Interrupt enabled while active, and vice versa when idle — refreshed every poll tick, the same bulk button-gating idiom gui_node.py's _set_busy() uses.

## 5.4 flight_executor_node.py

The companion-computer side of the ssh deploy backend — launched remotely by SSHDeployer via single_drone_flight.launch.py, not part of the host-side mission_node process. Reads one drone's rendered params file (automatically_declare_parameters_from_overrides=True, since the trajectory/params sub-dicts have a variable shape depending on the flight profile) and drives drone_commander.PX4Commander through that flight profile's trajectory in real time, at control_rate Hz.

SIGINT handling is intentional and non-standard: it replaces rclpy's default SIGINT handler so that a SIGINT (sent by SSHDeployer.interrupt(hard=False)) freezes the last commanded setpoint and keeps streaming it — the gentle-hold behavior — rather than exiting the process, since PX4 offboard needs a continuous setpoint stream or it drops out. Consequence: Ctrl+C no longer stops this node during manual testing; use SIGTERM (or SSHDeployer.interrupt(hard=True), which sends SIGKILL) to actually stop it.

Status and known limitation:  written but NOT yet validated against real hardware. Also: ROS 2 parameters cannot carry a nested list-of-lists, so a waypoints-type trajectory (whose points field is a list of [x, y, z, t] tuples) cannot currently flow through the rendered params YAML into this node's parameters. static_point and circle trajectories (flat scalar/array fields only) work correctly; waypoints is a follow-up once this path is validated against real hardware — see §13.

## 5.5 drone_commander.py (edited — additive only)

The only change to pre-existing code in this entire body of work. Fully backward compatible: every existing call site (formation_node.py) is unaffected because the new parameter defaults to today's exact behavior.

| Change | Detail |
| --- | --- |
| make_commander(..., control_scheme="position") | New keyword argument, default preserves prior behavior exactly. Passed through to PX4Commander; ignored by the sim backend (MockCommander only ever integrates a point mass toward a position target). |
| PX4Commander.set_target_velocity_enu(vx, vy, vz, yaw_rate) | New — velocity control scheme, ENU→NED converted the same way position already is. |
| PX4Commander.set_target_trajectory_enu(pos, vel, accel, yaw) | New — sets position plus optional velocity/acceleration feedforward. |
| PX4Commander.land() | New — sends VEHICLE_CMD_NAV_LAND. Used by mission_node's Terminate(land_first) handling for the local backend. |
| publish_heartbeat() | Now dispatches which OffboardControlMode booleans and TrajectorySetpoint fields are populated based on control_scheme; unset fields are set to NaN per TrajectorySetpoint.msg's documented “NaN means don't control” convention (previously they were implicitly zero). Behaviorally identical to before for control_scheme="position", the only mode formation_node.py ever uses. |

# 6. Launch Files & Configuration

| File | Purpose |
| --- | --- |
| launch/mission.launch.py | Starts mission_node. Declares a backend:= launch argument (default mock_ssh) that overrides every drone profile's default deploy backend for that run; loads config/mission_manager.yaml plus the installed config/drone_profiles/ and flight_profiles/ directories. |
| launch/mission_gui.launch.py | Starts mission_gui with the same config. |
| launch/single_drone_flight.launch.py | The static, checked-in launch file SSHDeployer runs remotely (§4.2) — takes one params_file:= argument and starts flight_executor_node with it. |

config/mission_manager.yaml — shared parameters for mission_node and mission_gui:

/**:
  ros__parameters:
    missions_root: "missions"               # resolved relative to the cwd `ros2 launch` runs from
    drone_profiles_dir: "config/drone_profiles"
    flight_profiles_dir: "flight_profiles"
    default_backend: "mock_ssh"             # local | ssh | mock_ssh - safe default, no hardware
    telemetry_rate: 5.0                     # Hz
    telemetry_stale_timeout: 3.0            # s (reserved for future staleness use)
    goal_timeout: 60.0                      # s - time allowed to reach 'streaming'

Two example drone profiles (config/drone_profiles/drone1.yaml, drone2.yaml) and two example flight profiles (flight_profiles/hover_low.yaml, circle_survey.yaml) are shipped, matching the real namespaces/system_ids already in config/drones.yaml, plus a README.md in each directory explaining the file format.

# 7. Tests

Five new test files were added under test/, matching the existing project's pytest style exactly (plain functions, bare/pytest.raises assertions, no unittest.TestCase, descriptive full-sentence names, tmp_path for filesystem tests) — 32 new test functions in total, all passing.

| File | Covers |
| --- | --- |
| test_flight_profiles.py | Loading + validation (unknown state/control_scheme raise ValueError), duplicate-name detection, and all three trajectory sample types including edge cases (zero period, clamped waypoints, empty waypoints list). |
| test_drone_profiles.py | Loading + validation (missing required field), duplicate-name detection, capability subset logic. |
| test_mission_store.py | create_mission() happy path and MissionExistsError; force=True overwrites mission.yaml but not README.md; save_mission() never touches README.md; list_missions() only lists directories with a mission.yaml; new_run_dir() never collides. |
| test_launch_gen.py | render_single_drone_params() output shape; write_params_yaml() produces valid, round-trippable /**: wildcard YAML. |
| test_deploy_backends.py | make_deployer() dispatches on the backend string correctly (including the '' → mock_ssh fallback); MockSSHDeployer's full happy path (precheck → deploy → phase progression via a fake timer → interrupt → terminate, asserting the fake log file exists at the returned path); LocalDeployer's lightweight contract. Uses a hand-written fake Node stand-in (just create_timer()) so these run with zero rclpy sourced. |

Deliberately not tested:  SSHDeployer's real subprocess/ssh/scp/rsync calls (no real network in CI), and flight_executor_node.py / mission_node.py's rclpy internals — consistent with the existing project's practice of not directly unit testing formation_node.py's or gui_node.py's rclpy internals either. These are instead verified by manual dry runs — see §8.

A real bug in deploy/backends.py surfaced while writing these tests: drone_commander.py was originally imported at the top of backends.py, and drone_commander.py itself imports rclpy/geometry_msgs at module scope — so importing backends.py (needed to test MockSSHDeployer, which has nothing to do with PX4) would have required a fully sourced ROS workspace, contradicting the “no ROS import at module scope” design goal. Fixed by moving that import inside LocalDeployer.deploy() (the one method that actually needs it), lazily — the same convention PX4Commander itself already uses for px4_msgs.

# 8. Verification Performed

Both packages were built (colcon build --packages-select formation_interfaces, then formation_interface) and the full test suite was run (colcon test --packages-select formation_interfaces formation_interface). 68 tests ran, 0 errors, 1 skipped (the pre-existing, already-skipped copyright check) — see §12 regarding the two style-check failures.

A live mission_node (backend:=mock_ssh) was then exercised directly, confirming:

Mission creation via mission_store.create_mission() produced the expected mission.yaml/README.md.

ros2 action/service/topic list showed /execute_flight, /terminate_flight, and /mission/drone_telemetry all correctly advertised.

Idle telemetry (before any Start) correctly reported unknown/idle defaults for a drone with no active deployer.

Sending an ExecuteFlight goal with no explicit flight_profile correctly resolved it from the mission's assignment map, and walked through deploying → copying → starting → streaming feedback exactly as designed, with nav_state/arming_state flipping to OFFBOARD/ARMED once streaming.

Telemetry after a successful Start correctly showed mission_phase="holding", the assigned mission_name, and simulated battery/mode values.

Requesting a flight profile that requires a capability the drone lacks (circle_survey needs "camera"; drone1 only has "gps") was correctly rejected with an explicit capability-mismatch message.

TerminateFlight with land_first=true correctly logged that the mock_ssh backend has no direct land command (expected — only local exposes one), then retrieved a real mock.ulog file at the returned log path.

A dedicated, deterministic rclpy test script confirmed the interrupt regression described in §5.2 is fixed: a goal reaching "streaming" stays open (does not self-complete) for at least a full second afterward, a cancel request against it correctly returns success=False / phase="held" with the deployer left holding, and the same drone_name is immediately free to accept a brand new goal afterward.

All modules were also confirmed to import cleanly with zero errors: mission_gui, mission_node, flight_executor_node, telemetry, deploy.backends, and deploy.launch_gen.

# 9. Other File Changes

| File | Change |
| --- | --- |
| setup.py | Three new console_scripts entries (mission_node, mission_gui, flight_executor_node); two new data_files globs for config/drone_profiles/*.yaml and flight_profiles/*.yaml (data_files globs do not recurse, so each new directory needs its own tuple). |
| package.xml | Added <exec_depend>python3-yaml</exec_depend> — PyYAML is already present system-wide (ROS 2's own launch system depends on it) but wasn't previously a declared dependency of this package, since no existing file imported it directly. |
| .gitignore | Added missions/*/runs/ — per-run params/logs are not tracked; mission.yaml and README.md remain trackable. |

# 10. How to Run It

source install/setup.bash
 
# terminal 1 - the mission manager node (safe default, no hardware required)
ros2 launch formation_interface mission.launch.py backend:=mock_ssh
 
# terminal 2 - the mission control GUI
ros2 launch formation_interface mission_gui.launch.py

In the GUI: type a mission name and description, click Create Mission; assign a flight profile to each drone row (dropdown is filtered by capability); click Start on a row or Start ALL; watch the telemetry table walk through deploying → copying → starting → streaming → holding with live simulated battery/mode; click Interrupt to gently hold a drone in place without ending its flight; click Terminate & Retrieve Logs to land (where supported) and pull its log file into missions/<name>/runs/<timestamp>/logs/<drone_name>/.

# 11. What Needs to Change in the Future

## 11.1 Before Flying on Real Hardware

SSHDeployer and flight_executor_node.py are written and structurally complete but have not been validated against a real companion computer — none was available during this work. Test against one real drone before trusting the ssh backend for anything real.

The waypoints trajectory type cannot reach flight_executor_node.py yet. ROS 2 parameters can't carry a nested list-of-lists, so trajectory.points (a list of [x, y, z, t] tuples) doesn't survive the rendered-params-YAML → node-parameters path. static_point and circle (flat scalar/array fields) work today. Needs either a different transport for that one field (e.g. ship it as a JSON string parameter and parse it on the other end) or restructuring the points list into parallel flat arrays that ROS parameters can carry.

land_first only sends a real AUTO_LAND command for the local backend. mission_node._maybe_land() only has a live PX4Commander to call land() on when the local backend is in use — that's the only backend running host-side. For ssh, landing needs to happen on the companion computer as part of its own terminate flow; flight_executor_node.py doesn't currently do this — on SIGINT it only holds. For mock_ssh there is nothing physical to land.

PX4 enum values are hand-copied in two places. NAV_STATE_NAMES in flight_profiles.py, and the enum constants in DroneTelemetry.msg, are both manually copied from px4_msgs/msg/VehicleStatus.msg. If the vendored px4_msgs is ever updated to a PX4 version with different enum values, both need updating by hand — nothing enforces they stay in sync with the vendored package.

## 11.2 Design / Scope Gaps Worth Revisiting

link_ok is a placeholder, not a real liveness check. mission_node._link_ok() currently just means “have we ever heard a pose for this drone” — not a genuine per-backend liveness signal (SSH reachability, DDS liveliness). It will not reliably flag a connection that drops mid-flight.

No hard/emergency interrupt is exposed end-to-end. SSHDeployer.interrupt(hard=True) (SIGKILL) already exists, but nothing in mission_gui.py or the ExecuteFlight action goal currently exposes a way to request it — only gentle interrupt is wired up, matching the requirement as given. Worth adding a real E-stop path if one is ever needed.

config/drone_profiles/*.yaml and config/drones.yaml overlap and are not synced. Both describe the same underlying facts (namespace, system_id, pose topic) for two different subsystems — the formation stack and the mission stack respectively. A namespace change today means editing both files by hand; consider generating one from the other, or merging the two config surfaces, the next time either changes.

features.record_bag does nothing yet. It exists as a toggle in mission.yaml but nothing reads it — no ros2 bag integration has been wired up.

Rich per-field error detail (FailsafeFlags) was scoped out of v1. DroneTelemetry only has the rollup bools failsafe_active / gcs_connection_lost / pre_flight_checks_pass. The error_message string field exists for richer detail but nothing populates it yet.

mission_gui.py has no map view and no in-GUI profile editing. By design, to keep v1 scope down — profiles are hand-edited YAML, picked up via the “reload profiles” button. Revisit if that friction becomes a real problem in practice.

missions_root resolves relative to the current working directory. Not an absolute path by default — documented with a “>>> CHANGE <<<” comment in config/mission_manager.yaml, the same convention drones.yaml already uses for its topic list, but easy to get wrong if ros2 launch isn't run from the repo root.

## 11.3 Not a Regression, But Worth Knowing

Pre-existing repo-wide condition:  colcon test's flake8/pep257 checks fail on this package both before and after this work — verified directly against a clean baseline (git stash -u before, rerun, compare pass/fail counts: identical). gui_node.py, formation_node.py, interface_node.py, monitor_gui.py, and mock_optitrack.py all fail the same way already (hundreds of quote-style and import-order findings each), unrelated to anything in this document. The new files match the existing code's actual style (double-quoted strings, the same docstring conventions); fixing the underlying ament_flake8 config mismatch is a separate, unrelated cleanup task, not something introduced here.

# 12. Appendix: Full File Listing

## New files

| Path | One-line description |
| --- | --- |
| src/formation_interfaces/action/ExecuteFlight.action | Per-drone flight action. |
| src/formation_interfaces/srv/TerminateFlight.srv | Land + retrieve logs service. |
| src/formation_interfaces/msg/DroneTelemetry.msg | Live per-drone status message. |
| src/formation_interface/formation_interface/flight_profiles.py | Flight profile dataclass, loader, trajectory sampler. |
| src/formation_interface/formation_interface/drone_profiles.py | Drone profile dataclass, loader, capability check. |
| src/formation_interface/formation_interface/mission_store.py | Mission dataclass, anti-overwrite create/save/list. |
| src/formation_interface/formation_interface/deploy/__init__.py | Package marker + docstring. |
| src/formation_interface/formation_interface/deploy/launch_gen.py | Params-file rendering (pure functions). |
| src/formation_interface/formation_interface/deploy/backends.py | Local/SSH/MockSSH deployer classes + factory. |
| src/formation_interface/formation_interface/telemetry.py | TelemetrySample + PX4TelemetryWatcher. |
| src/formation_interface/formation_interface/mission_node.py | ExecuteFlight action server + TerminateFlight service. |
| src/formation_interface/formation_interface/mission_gui.py | Tkinter mission control panel. |
| src/formation_interface/formation_interface/flight_executor_node.py | Companion-computer-side flight executor (phase 2). |
| src/formation_interface/launch/mission.launch.py | Launches mission_node. |
| src/formation_interface/launch/mission_gui.launch.py | Launches mission_gui. |
| src/formation_interface/launch/single_drone_flight.launch.py | Remote launch target for the ssh backend. |
| src/formation_interface/config/mission_manager.yaml | Shared params for mission_node/mission_gui. |
| src/formation_interface/config/drone_profiles/drone1.yaml, drone2.yaml | Example drone profiles. |
| src/formation_interface/config/drone_profiles/README.md | Format explanation. |
| src/formation_interface/flight_profiles/hover_low.yaml, circle_survey.yaml | Example flight profiles. |
| src/formation_interface/flight_profiles/README.md | Format explanation. |
| src/formation_interface/test/test_flight_profiles.py | 14 tests. |
| src/formation_interface/test/test_drone_profiles.py | 5 tests. |
| src/formation_interface/test/test_mission_store.py | 8 tests. |
| src/formation_interface/test/test_launch_gen.py | 2 tests. |
| src/formation_interface/test/test_deploy_backends.py | 3 tests. |
| src/formation_interface/MISSION_MANAGER.md | Markdown version of this document. |

## Edited files (all additive/backward compatible)

| Path | Change |
| --- | --- |
| src/formation_interface/formation_interface/drone_commander.py | control_scheme kwarg + velocity/trajectory setpoints + land(). |
| src/formation_interface/setup.py | New console_scripts + data_files globs. |
| src/formation_interface/package.xml | Added python3-yaml dependency. |
| src/formation_interfaces/CMakeLists.txt | Registered the three new interface files. |
| .gitignore | Ignore missions/*/runs/. |